Windows 95

Installing the AdvanSys Universal Driver

Normal installation of the AdvanSys Universal Windows 95 Driver for VL Single Channel and ISA

The procedure for installing the AdvanSys Universal Windows 95 driver is nearly the same for Single Channel VL and ISA adapters.  These adapters are not Plug and Play, so they must be configured by the user.

Step by step installation checklist:

1) Boot Windows 95 into Normal Mode (not Safe Mode).  In some cases,  it is not possible to boot into Normal Mode.  If this is due to an aborted attempt to install the AdvanSys Universal Windows 95 driver, the user must first boot Safe Mode, then remove the driver from the list of devices in My Computer/Control Panel/System/Device Manager.  After removing all occurrences of the driver, the user should be able to boot into Normal Mode.

2) Select the Add New Hardware Wizard from My Computer/Control Panel.

3) Click Next.

4) Click No, then Next.  Do Not allow Windows to search for new hardware.

5) Select SCSI controllers from near the bottom of the list of hardware types, then click Next.

6) Click Have Disk.

7) Insert the AdvanSys Universal Drivers Diskette in drive A:, enter A:\WIN95, then click OK.

8) Click the Model of AdvanSys adapter being installed (VL or ISA), then click Next.

9) Write down the Resource Settings Windows 95 has assigned to the adapter: DMA channel (ISA), I/O Range and IRQ.

10) Click Next to clear the Resources screen, Windows will copy the AdvanSys Universal Windows 95 driver.

11) Click Finish.

12) Click Yes,  the computer will shut down.

13) When the message “It’s now safe to turn off your computer” is displayed, power down the computer.

14A) If the adapter has an onboard BIOS,  power up the computer and press CTRL-A when the AdvanSys adapter begins its device scan.

14B) If there is no onboard BIOS, insert a diskette containing the AdvanSys configuration utility, then power up the computer.

15) Configure the adapter using the Resource Settings from step 9.

16) Remove and floppies and be sure to “Write setup & exit” after changing Adapter Configuration, the computer will reboot into Windows 95.

17) Go to the Device Manager by clicking My Computer/Control Panel/System/Device Manager.

18) Click the AdvanSys ISA (or VL) SCSI Host Adapter Driver icon, then select Properties.

19) Go to the Device Manager by clicking My Computer/Control Panel/System/Device Manager.

20) Click the AdvanSys ISA (or VL) SCSI Host Adapter Driver icon, then select Properties.

21) Verify that “This device is working properly” is displayed in the Device status box.

Installation of the AdvanSys Universal Windows 95 driver is now complete.

Installing the AdvanSys Universal Windows 95 Driver for VL Single Channel and ISA when the user needs to set the I/O Port Address or IRQ.

The procedure for installing the AdvanSys Universal Windows 95 driver is nearly the same for Single Channel VL and ISA adapters.  These adapters are not Plug and Play, so they must be configured by the user.  This discussion assumes that the hardware has been installed and configured correctly, and operates properly with ADVASPI.SYS.

Step by step installation checklist:

1) Note the adapter settings for I/O Port and IRQ.  These values may be obtained from the BIOS CTRL-A feature, or from a separate utility if there is no BIOS present.

2) Boot Windows 95 into Normal Mode (not Safe Mode).  In some cases,  it is not possible to boot into Normal Mode.  If this is due to an aborted attempt to install the AdvanSys Universal Windows 95 driver, the user must first boot Safe Mode, then remove the driver from the list of devices in My Computer/Control Panel/System/Device Manager.  After removing all occurrences of the driver, the user should be able to boot into Normal Mode.

3) Select the Add New Hardware Wizard from My Computer/Control Panel.

4) Click Next.

5) Click No, then Next.  Do Not allow Windows to search for new hardware.

6) Select SCSI controllers from near the bottom of the list of hardware types, then click Next.

7) Click Have Disk.

8) Insert the AdvanSys Universal Drivers Diskette in drive A:, enter A:\WIN95, then click OK.

9) Click the Model of AdvanSys adapter being installed (VL or ISA), then click Next.

10) Click Next to clear the Resources screen, Windows will copy the AdvanSys Universal Windows 95 driver.

IMPORTANT:  After the following step, DO NOT click Yes to shut down.

11) Click Finish.

12) Click No.  Do Not Shut Down the computer yet!

13) Go to the Device Manager by clicking My Computer/Control Panel/System/Device Manager.

14) Click the AdvanSys ISA (or VL) SCSI Host Adapter Driver icon, then select Properties.

15) Select Resources, then Set Configuration Manually.

16) If the Input/Output Range or Interrupt Request is not the same as the card as noted in step 1, double click on that Resource Type.

17) Click on the Up and/or Down Arrows as appropriate to select the correct value.

18) Ensure that  “No devices are conflicting.” Is displayed in the Conflict information box.

19) Click Ok.  Repeat steps 16 through 18 for the other Resource as appropriate.

20) Click Ok to exit Resources.

21) Click Yes... (Shut down the computer now), and re-boot Windows 95 Normal Mode.

22) Go to the Device Manager by clicking My Computer/Control Panel/System/Device Manager.

23) Click the AdvanSys ISA (or VL) SCSI Host Adapter Driver icon, then select Properties.

24) Verify that “This device is working properly” is displayed in the Device status box.

Installation of the AdvanSys Universal Windows 95 driver is now complete.

Installing the AdvanSys Universal Windows 95 Driver for Dual Channel VL Adapter

The AdvanSys Dual Channel VL Adapter presents itself to Windows 95 as two independent single channel VL adapters.  To install the AdvanSys Universal Windows 95 Driver, the user merely follows the checklist for a Single channel VL adapter, and then performs the checklist a second time for the other channel.

Installing the AdvanSys Universal Windows 95 Driver for PCI and EISA

When an AdvanSys PCI or EISA adapter is added to a functional Windows 95 system, Windows 95 will detect the presence of the adapter upon the first boot after installing the adapter.  Windows 95 will display the “New Hardware Found” dialog.  When Windows 95 is installed for the first time on a system that contains an AdvanSys PCI or EISA adapter, this dialog is not displayed.  The following checklist covers both possibilities.

Step by step installation checklist:

1) If the “New Hardware Found” dialog is NOT displayed, skip to step 6.

2) Click “Driver ffrom disk provided by hardware manufacturer”.

3) Click OK.

4) Insert the AdvanSys Universal Drivers Diskette in drive A:, enter A:\WIN95 then click OK, Windows will copy the AdvanSys Universal Windows 95 driver.

5) Skip to step 16.

If the New Hardware Found dialog is NOT displayed:

6) Go to the Device Manager by clicking My Computer/Control Panel/System/Device Manager.

7) Double Click on Other devices.

8) Click PCI SCSI Bus Controller, then select Properties.

9) Click the Driver tab at the top.

10) Click Change Driver.

11) Select SCSI Controllers from near the bottom of the list, Click OK.

12) Click Have Disk.

13) Insert the AdvanSys Universal Drivers Diskette in drive A:, enter A:\WIN95 then click OK.

14) Select AdvanSys PCI (EISA) SCSI Host Adapter Driver and Click OK.

15) Click OK, Windows will copy the AdvanSys Universal Windows 95 driver.

16) Click Yes to restart the computer.

17) Go to the Device Manager by clicking My Computer/Control Panel/System/Device Manager.

18) Click the AdvanSys PCI (EISA) SCSI Host Adapter Driver icon, then select Properties.

19) Verify that “This device is working properly” is displayed in the Device status box.

Installation of the AdvanSys Universal Windows 95 driver is now complete.

--DRB 09/15/95