%OP%DESystem Calls Reference

%OP%BON

%OP%AMM

%OP%HM3

%OP%FM3

%OP%BM0

%OP%LM10

%OP%HE/@T@

%OP%FO//Z88 Developer's Notes V3, @P@//

%CO:A,5,74%OS_UST, update small timer

RST 20H, DEFB $78

IN:

OUT:

Notes:

Bugs:

%P0%

%CO:B,5,69%

BC = new timer value

Fc = 0

Fz = 0, old time value non-zero

Fz = 1, old time value zero

BC = old time value

Internal operating system usage.

The timer latches at $0000. An initial value of $FFFF will not be

decremented. The timer decrements once per centiseconds.

This code uses same timer as OS_TIN and OS_DLY.

%CO:C,5,64%%CO:D,5,59%%CO:E,5,54%%CO:F,5,49%